Section Seven Fundamentals of computer organisation and architecture

Practice Questions (with answers)

- Processors use the fetch-execute cycle.

  - Describe in full sentences how the fetch-execute cycle works in relation to the main registers: Current Instruction Register (CIR), Memory Address Register (MAR), Memory Buffer Register (MBR), Program Counter (PC) and Status Register (SR).

- The Current Instruction Register (CIR) stores the instruction that is currently being executed by the processor.

- The Program Counter (PC) stores the memory location of the next instruction that will be needed by the processor.

- The Memory Buffer Register (MBR), also known as the Memory Data Register (MDR) holds the data that has just been read from or is about to be written to main memory.

- The Memory Address Register (MAR) stores the memory location where data in the MBR is about to be written to or read from.

- The Status Register (SR) keeps track of the status of various parts of the computer – for example, if an overflow error has occurred during an arithmetic operation.

  - Explain the three types of buses used in a computer and what they do during the fetch-execute cycle.

Data bus: This bus passes the instructions and data that comprise a computer program back and forth between the processor and memory as the program is run.

Address bus: This bus only goes in one direction – from the processor into memory. All the instructions and data that a processor needs to carry out a task are stored in memory. The address bus is used by the processor and carries the memory address of the next instruction or data item.

Control bus: This is a bi-directional bus that sends control signals to the registers, and to the data and address buses.

  - An interrupt will suspend execution of the current program by the processor. What is an interrupt?
An interrupt is a signal (e.g. from a hardware device) that stops the processor from carrying out its current instruction in order to deal with another task.

- Explain the difference between the Von Neumann and Harvard architectures.
Von Neumann stores data and instructions together in memory. Harvard stores data and instructions in separate parts of memory.

- Write assembly language instructions that would perform the same task as the pseudo-code below. Use the registers r1 and r2 to store the variables A and B.

If A = 1 THEN
  B     2
ELSE
  A    A + 1
ENDIF

CMP r1, #1 		compare N to 1

BNE else 		jump if not equal to label else
MOV r2, #2		move the value 2 to variable B

B endif		jump to end of statement block

else

ADD r1,r1, #1	add the value of 1 onto N

endif

- Two alternatives for data storage are hard disk drives (HDD) and solid state disks (SSD).

  - Describe the key principles of how each work.
The key features of HDD are: metallic disk or disks, hermetically sealed, read/write heads, disk split into sectors and tracks, mechanical operation in terms of spinning and moving the head to read/write the data.
The key features of SSD are: using programmable memory so entirely electronic; uses semiconductors split into blocks; uses floating gate transistors.

  - State two advantages of HDD over SSD.
Cheaper to manufacture, larger volumes of data can be stored.

  - State two advantages of SSD over HDD.
Access times are much faster, low latency rates and low power consumption.

- Explain how the following can lead to faster execution of programs:

  - Increasing the clock speed
Clock speed is related to the number of instructions per second that a processor can carry out. Increasing the clock speed increases the number of instructions and therefore leads to faster execution of programs.

  - Modifying the width of the data bus
Making the data bus wider will allow more data to be passed around the processor within a given time frame.

  - Utilising cache memory
Cache memory is separate from main memory and smaller. It sits between main memory and the processor. Therefore it can be accessed more quickly. If part of a program is put into cache, the program will run faster.

- ‘Police officers will be able to respond faster to emergency calls and spend more time on the street if they make better use of modern technology.’ Why might some people object to police officers using new technology? 
Many people are concerned about a lack of privacy and too much control being exercised by the government. Consequently they may object to personal data being collected and used by the police. This could include simple information such as name or address, but may include details of previous convictions or accusations or even biological data such as DNA or other samples.

- Name the most suitable storage medium for each of the following.

  - Backing up a 30 Kb email message
memory stick

  - Backing up 2 Gb of data
memory stick or optical disk

  - Distributing a software package requiring 500 Mb of storage space.
optical disk