Chapter 34 – The processor instruction set and addressing modes

task Questions (with answers)

- Explain the difference between direct and immediate addressing.

The address used in a direct address points to where the required data is stored.

The address in an indirect address points to a memory location that in turn contains the address where the data that is needed is stored.

- What is an operation code?

An operation code is a key that links to a list of instructions. The CPU uses the op-code to look up what to do in the instruction set.

- Some, but not all, operation codes are followed by one or more operands.

- What is an operand?

An operand is a number or code that needs to be used or manipulated in some way.

- Why does the number of operands vary from one operation code to the next?

The number of operands needed after an op-code is dependent on what the instruction is. A comparison would need two operands, while a save instruction would only need one.

- Give an example to illustrate each of the following types of operation code:

- data transfer

the process of moving data to/from RAM and the CPU

- arithmetic operation

addition, subtraction or some other calculation

- logical operation

uses logic gates, such as AND and OR, to carry comparisons

- branch operation

allows a programmer to move to another section of the code, possibly because of a comparison

- shift operation

A shift operation moves all the bits in a byte either left or right. It can be used either to extract the value of a specific bit or when the CPU has been asked to multiply two numbers together.

- Write assembly language instructions to create a counter that counts from 0 to 10 and then halts.

LDR r1, 0

Loop	CMP r1, #10

BNE

ADD r1, #1

B (Loop)

- Write assembly language for the following pseudo code. X should be stored as a variable in memory:

if x > 0

then x = x -1

End if

Loop	CMP r1, #0 		compare X (in register 1) to 0

BLT (Halt) 				jump to halt if less than 0

SUB r1, #1				subtract the X by 1

B (Loop)		branch to the start of the loop

Halt