Chapter 37 –  Communication basics

task Questions (with answers)

- Identify two scenarios where a computer might use:

  - serial connections
Connecting workstations to servers in a network
Connecting peripherals to a computer e.g. scanner, printer

  - parallel connections.
Connecting memory to registers in a processor e.g. buses
Connecting main memory to cache memory

- What are the advantages of USB connections over traditional serial connections?
They are widely compatible with a massive range of devices and peripherals.
They use very little power.
They are physically small, enabling more to be fitted onto a computer.
They enable high speeds of data transmission.
Power can be supplied to devices through the USB port.

- Explain the difference between synchronous and asynchronous data transmission.
Synchronous means ‘occurring at the same time’ or ‘having the same speed’. In the context of transmissions this means that the two devices which are communicating will synchronise their transmission signals. Using the system clock, the computer sending the data will control the transmission rate to be in time with the device or computer receiving the signal. If the two devices are not synchronised then data could be lost during transmission. Once they are synchronised, the two devices can send and receive data without need for any further information.
Asynchronous transmission does not require the permanent synchronisation of the sender’s and receiver’s system clocks. Instead, it synchronises only for the duration of the transmission by sending additional bits of information called start and stop bits.

- Explain the relationship between bit rate, baud rate and bandwidth.
Bit rate is the actual amount of data being transmitted within a given time frame.
Baud rate is another term used to describe the speed at which data can be transmitted. One baud represents one electronic state change per second where several bits can be encoded and sent on each state change.

Bandwidth is the amount of data that a transmission medium (e.g. cable) is capable of sending within a given time frame.

- Why are protocols needed in computing?

Protocols are sets of rules that everyone must stick to. They are important in computing as there are so many different manufacturers of hardware, software and communication systems that there needs to be some agreed standards which allow all of these various aspects to work together. For example, the www could not exist unless everyone agreed on a standard way of transmitting web pages.

- What is latency and what are the main causes?
Latency is the general term used to explain the time delay that occurs when any component within a computer system is responding to an instruction. In simple terms it is the time it takes from instructing a process to occur and the results being seen.

- Propagation latency – the amount of time it takes for a logic gate within a circuit to transmit the data.

- Transmission latency – the amount of time it takes to pass through a particular communication medium. For example, fibre optic would have a lower latency than copper cable.

- Processing latency – the amount of time it takes data to pass around a network, depending on how many servers or devices it has to pass through.